Hasil Tes Remote

Dpad Values:

Left: 0x08

Down: 0x02

Right: 0x04

Up: 0x01

Button Values

Triangle: 0x0008

Circle: 0x0002

Cross: 0x0001

Square: 0x0004

L3: 0x0100

R3: 0x0200

What does above image mean?

X-Axis range: -508 – 512 (left to right)

Y-Axis range: -508 – 512 (up to down)

If the L or R axis is steered diagonally, it will combine both X and Y axis values, creating cartesian coordinate-like result.

What about if there are 2 buttons pressed at the same time?

Simply add both values together. Example: Pressing Dpad Left and Down will result in 0x0A, which is the result of 0x08 (left) + 0x02 (right).

Additional Notes:

The properties for controller have more than 15 attributes, 4 of which are the dpad, buttons, L axis, and R axis. There are others like throttle, gyro, x, y, z, accel, etc. Right now, idk how to use them yet.

Code (In Progress)

#include <Bluepad32.h>

ControllerPtr myControllers[BP32_MAX_GAMEPADS];

void onConnectedController(ControllerPtr ctl) {

bool foundEmptySlot = false;

for (int i = 0; i < BP32_MAX_GAMEPADS; i++) {

if (myControllers[i] == nullptr) {

Serial.printf("CALLBACK: Controller is connected, index=%d\n", i);

// Additionally, you can get certain gamepad properties like:

// Model, VID, PID, BTAddr, flags, etc.

ControllerProperties properties = ctl->getProperties();

Serial.printf("Controller model: %s, VID=0x%04x, PID=0x%04x\n", ctl->getModelName().c_str(), properties.vendor_id,

properties.product_id);

myControllers[i] = ctl;

foundEmptySlot = true;

break;

}

}

if (!foundEmptySlot) {

Serial.println("CALLBACK: Controller connected, but could not found empty slot");

}

}

void onDisconnectedController(ControllerPtr ctl) {

bool foundController = false;

for (int i = 0; i < BP32_MAX_GAMEPADS; i++) {

if (myControllers[i] == ctl) {

Serial.printf("CALLBACK: Controller disconnected from index=%d\n", i);

myControllers[i] = nullptr;

foundController = true;

break;

}

}

if (!foundController) {

Serial.println("CALLBACK: Controller disconnected, but not found in myControllers");

}

}

void dumpGamepad(ControllerPtr ctl) {

Serial.printf(

"idx=%d, dpad: 0x%02x, buttons: 0x%04x, axis L: %4d, %4d, axis R: %4d, %4d, brake: %4d, throttle: %4d, "

"misc: 0x%02x, gyro x:%6d y:%6d z:%6d, accel x:%6d y:%6d z:%6d\n",

ctl->index(),        // Controller Index

ctl->dpad(),         // D-pad

ctl->buttons(),      // bitmask of pressed buttons

ctl->axisX(),        // (-511 - 512) left X Axis

ctl->axisY(),        // (-511 - 512) left Y axis

ctl->axisRX(),       // (-511 - 512) right X axis

ctl->axisRY(),       // (-511 - 512) right Y axis

ctl->brake(),        // (0 - 1023): brake button

ctl->throttle(),     // (0 - 1023): throttle (AKA gas) button

ctl->miscButtons(),  // bitmask of pressed "misc" buttons

ctl->gyroX(),        // Gyro X

ctl->gyroY(),        // Gyro Y

ctl->gyroZ(),        // Gyro Z

ctl->accelX(),       // Accelerometer X

ctl->accelY(),       // Accelerometer Y

ctl->accelZ()        // Accelerometer Z

);

}

void processGamepad(ControllerPtr ctl) {

// There are different ways to query whether a button is pressed.

// By query each button individually:

//  a(), b(), x(), y(), l1(), etc...

if (ctl->a()) {

static int colorIdx = 0;

// Some gamepads like DS4 and DualSense support changing the color LED.

// It is possible to change it by calling:

switch (colorIdx % 3) {

case 0:

// Red

ctl->setColorLED(255, 0, 0);

break;

case 1:

// Green

ctl->setColorLED(0, 255, 0);

break;

case 2:

// Blue

ctl->setColorLED(0, 0, 255);

break;

}

colorIdx++;

}

if (ctl->b()) {

// Turn on the 4 LED. Each bit represents one LED.

static int led = 0;

led++;

ctl->setPlayerLEDs(led & 0x0f);

}

if (ctl->x()) {

ctl->playDualRumble(0 /* delayedStartMs */, 250 /* durationMs */, 0x80 /* weakMagnitude */,

0x40 /* strongMagnitude */);

}

// Another way to query controller data is by getting the buttons() function.

// See how the different "dump*" functions dump the Controller info.

if (ctl->dpad() == 0x08) {  // Left D-pad

Serial.println("D-pad Left pressed");

}

else if (ctl->dpad() == 0x02) {  // Down D-pad

Serial.println("D-pad Down pressed");

}

else if (ctl->dpad() == 0x04) {  // Right D-pad

Serial.println("D-pad Right pressed");

}

else if (ctl->dpad() == 0x01) {  // Up D-pad

Serial.println("D-pad Up pressed");

}

if (ctl->dpad() == (0x08 | 0x02)) {  // Left + Down (0x08 + 0x02 = 0x0A)

Serial.println("D-pad Left + Down pressed");

}

else if (ctl->dpad() == (0x08 | 0x04)) {  // Left + Right (0x08 + 0x04 = 0x0C)

Serial.println("D-pad Left + Right pressed");

}

else if (ctl->dpad() == (0x02 | 0x04)) {  // Down + Right (0x02 + 0x04 = 0x06)

Serial.println("D-pad Down + Right pressed");

}

else if (ctl->dpad() == (0x01 | 0x08)) {  // Up + Left (0x01 + 0x08 = 0x09)

Serial.println("D-pad Up + Left pressed");

}

else if (ctl->dpad() == (0x01 | 0x02)) {  // Up + Down (0x01 + 0x02 = 0x03)

Serial.println("D-pad Up + Down pressed");

}

else if (ctl->dpad() == (0x01 | 0x04)) {  // Up + Right (0x01 + 0x04 = 0x05)

Serial.println("D-pad Up + Right pressed");

}

// Check for all directions pressed together (0x08 + 0x02 + 0x04 + 0x01 = 0x0F)

else if (ctl->dpad() == (0x08 | 0x02 | 0x04 | 0x01)) {  // All directions

Serial.println("All D-pad directions pressed");

}

dumpGamepad(ctl);

}

void processControllers() {

for (auto myController : myControllers) {

if (myController && myController->isConnected() && myController->hasData()) {

processGamepad(myController);

}

}

}

// Arduino setup function. Runs in CPU 1

void setup() {

Serial.begin(115200);

Serial.printf("Firmware: %s\n", BP32.firmwareVersion());

const uint8_t* addr = BP32.localBdAddress();

Serial.printf("BD Addr: %2X:%2X:%2X:%2X:%2X:%2X\n", addr[0], addr[1], addr[2], addr[3], addr[4], addr[5]);

// Setup the Bluepad32 callbacks

BP32.setup(&onConnectedController, &onDisconnectedController);

BP32.forgetBluetoothKeys();

BP32.enableVirtualDevice(false);

}

void loop() {

bool dataUpdated = BP32.update();

if (dataUpdated)

processControllers();

delay(150);

}